Interpretation and Use of Binary RSC-11-10A Data

This document describes and illustrates extraction of values from binary files generated according to the NASA Deep Space Network (DSN) RSC-11-10A Software Interface Specification (SIS).  In this document, RSC-11-10A is referenced as:

[1] Document 820-013 (Rev. A), DSN System Requirements, Detailed Interface Design, 
RSC-11-10A, Voyager Neptune ODR Agreement, effective date July 1988 (PDS logical identifier urn:nasa:pds:radiosci.documentation:dsn.rsc-11-10a:1988-07-01)

RSC-11-10A is one of several modules within DSN 820-013 that governed generation of open loop radio science raw data products over about two decades starting in the late 1970s.  RSC-11-10A specifies the format and content of the open loop radio science files collected using the 70-m NASA Deep Space Network antenna near Canberra, Australia, during the Voyager 2 encounter with Neptune.  Variations of this format, used for collection of data during the same encounter at Parkes Observatory (Australia) and the Usuda Tracking Station (Japan), are described separately.

Data Overview:

Digital open loop data were the primary inputs for the study of radio occultations by the neutral atmosphere and ionosphere of Neptune and by the ionosphere of Triton.

The data were generated by the Spectrum Processing Assembly (SPA) of the Deep Space Communications Complex (DSCC) Spectrum Processing Subsystem (DSP), using radio science software of the DSP, designated “DSP-R”.

During the Neptune encounter, S- and X-band receiver outputs in right-circular polarization (RCP) from the 70-m antenna (DSS-43) at the DSCC near Canberra, Australia, were written to tape at 100,000 8-bit (real) samples per second from each of the two receiver channels.  The X-RCP receiver output was sampled alternately by analog-to-digital converters (ADCs) AD1 and AD3.  The S-RCP receiver output was sampled alternately by AD2 and AD4.  Each ADC operated at 50,000 samples per second; AD2 and AD4 were delayed by 10 ms relative to AD1 and AD3, yielding an effective rate of 100,000/s for each receiver.

Table 1 shows the Neptune occultation timeline, significant events, and corresponding tape numbers (column 6).  Table 2 provides the same information with events before, during, and after the Triton occultation.

The data were originally distributed on Computer-Compatible Tapes (CCTs).  In the mid-1990s the tape data were copied to CD-ROM volumes, and in 2020 the CD-ROMs were copied to a laptop computer.  Subsequent handling has exclusively used the electronic (laptop) files.  Throughout, [1] has remained the descriptive document of the Canberra data.

| Event Time (UTC) | Event | Tape Start (s) | Tape State (UTC) | Tape Stop (UTC) | Tape NC | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 09302 | 02:35:02 | 02:38:03 | 0590 | Test |
|  |  | 09722 | 02:42:02 | 02:50:00 | 0591 | Test |
|  |  | 11522 | 03:12:02 | 03:20:02 | 0592 | Test |
|  |  | 12002 | 03:20:02 | 03:28:01 | 0593 | Test |
| 06:40:02 | Begin recording of Neptune occultation | 24002 | 06:40:02 | 06:48:02 | 0594 |  |
|  |  | 24482 | 06:48:02 | 06:56:02 | 0595 |  |
| 06:59:09 07:01:02 07:02:02 | Inbound ring plane crossing S-band telemetry OFF X-band telemetry OFF | 24962 | 06:56:02 | 07:04:02 | 0596 |  |
| 07:06:02 07:11:25 | Start roll to Canopus End roll to Canopus | 25442 | 07:04:02 | 07:12:02 | 0597 |  |
| 07:14:52 | Begin Mini-ASCAL | 25922 | 07:12:02 | 07:20:02 | 0598 |  |
| 07:22:53 07:26:03 | End Mini-ASCAL TWNC OFF | 26402 | 07:20:02 | 07:28:02 | 0599 |  |
|  |  | 26882 | 07:28:02 | 07:36:02 | 0600 |  |
| 07:42:03 | TWNC ON | 27362 | 07:36:02 | 07:44:02 | 0601 |  |
|  |  | 27842 | 07:44:02 | 07:52:02 | 0602 |  |
|  |  | 28322 | 07:52:02 | 08:00:02 | 0603 |  |
| 08:07:57 | Begin limbtrack maneuver | 28802 | 08:00:02 | 08:08:02 | 0604 |  |
|  |  | 29282 | 08:08:02 | 08:16:02 | 0605 |  |
|  |  | 29762 | 08:16:02 | 08:24:02 | 0606 |  |
|  |  | 30242 | 08:24:02 | 08:32:02 | 0607 |  |
|  |  | 30722 | 08:32:02 | 08:40:02 | 0608 |  |
|  |  | 31202 | 08:40:02 | 08:48:02 | 0609 |  |
|  |  | 31682 | 08:48:02 | 08:56:02 | 0610 |  |
| 08:56:54 | End limbtrack maneuver | 32162 | 08:56:02 | 09:04:02 | 0611 |  |
|  |  | 32642 | 09:04:02 | 09:12:02 | 0612 |  |
|  |  | 33122 | 09:12:02 | 09:20:02 | 0613 |  |
| 09:20:59 09:24:30 | Outbound ring plane crossing Begin Mini-ASCAL | 33602 | 09:20:02 | 09:28:02 | 0614 |  |
| 09:32:30 | End Mini-ASCAL | 34082 | 09:28:02 | 09:36:02 | 0615 |  |
| 09:38:04 09:39:04 09:39:54 09:43:42 | X-band telemetry ON S-band telemetry ON Begin IMC End IMC | 34562 | 09:36:02 | 09:44:02 | 0616 |  |
| 09:51:40 | Begin roll to Alkaid | 35042 | 09:44:02 | 09:52:02 | 0617 |  |
| 10:00:00 | Return to normal subreflector focusing | 35522 | 09:52:02 | 10:00:02 | 0618 |  |
| 10:02:24 10:05:16 10:06:61 | End roll to Alkaid Real-time TWNC OFF Begin IMC | 36002 | 10:00:02 | 10:08:02 | 0619 |  |
|  |  | 36482 | 10:08:02 | 10:16:02 | 0620 |  |
|  |  | 36962 | 10:16:02 | 10:24:02 | 0621 |  |
| 10:30:04 | TWNC ON | 37442 | 10:24:02 | 10:32:02 | 0622 |  |
| 10:32:28 10:36:06 | End IMC Begin NIMC | 37922 | 10:32:02 | 10:40:02 | 0623 |  |
|  |  | 38402 | 10:40:02 | 10:48:02 | 0624 |  |
| 10:55:01 | End recording of Neptune occultation | 38882 | 10:48:02 | 10:55:01 | 0625 |  |

Table 1. Voyager 2 Neptune occultation time line with open loop data tape numbers (column 6).

| Event Time (UTC) | Event | Tape Start (s) | Tape State (UTC) | Tape Stop (UTC) | Tape NC | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 12:20:02 | Begin recording of Triton occultation | 44402 | 12:20:02 | 12:28:02 | 0626 |  |
|  |  | 44882 | 12:28:02 | 12:36:02 | 0627 |  |
|  |  | 45362 | 12:36:02 | 12:44:02 | 0628 |  |
|  |  | 45842 | 12:44:02 | 12:52:02 | 0629 |  |
|  |  | 46322 | 12:52:02 | 13:00:02 | 0630 |  |
|  |  | 46802 | 13:00:02 | 13:08:02 | 0631 |  |
|  |  | 47282 | 13:08:02 | 13:16:02 | 0632 |  |
|  |  | 47762 | 13:16:02 | 13:24:02 | 0633 |  |
| 13:28:45 13:29:45 | S-band telemetry OFF X-band telemetry OFF | 48242 | 13:24:02 | 13:32:02 | 0634 |  |
|  |  | 48722 | 13:32:02 | 13:40:02 | 0635 |  |
| 13:45:26 | Begin Triton occultation | 49202 | 13:40:02 | 13:48:02 | 0636 |  |
| 13:48:26 | End Triton occultation | 49682 | 13:48:02 | 13:56:02 | 0637 |  |
|  |  | 50162 | 13:56:02 | 14:04:02 | 0638 |  |
| 14:05:46 14:06:46 | X-band telemetry ON S-band telemetry ON | 50642 | 14:04:02 | 14:12:02 | 0639 |  |
|  |  | 51122 | 14:12:02 | 14:20:02 | 0640 |  |
|  |  | 51602 | 14:20:02 | 14:28:02 | 0641 |  |
|  |  | 52082 | 14:28:02 | 14:36:02 | 0642 |  |
|  |  | 52562 | 14:36:02 | 14:44:02 | 0643 |  |
|  |  | 53042 | 14:44:02 | 14:52:02 | 0644 |  |
|  |  | 53522 | 14:52:02 | 15:00:02 | 0645 |  |
|  |  | 54002 | 15:00:02 | 15:08:02 | 0646 |  |
|  |  | 54482 | 15:08:02 | 15:16:02 | 0647 |  |
|  |  | 54962 | 15:15:02 | 15:24:00 | 0648 |  |
|  |  | 55442 | 15:24:02 | 15:32:02 | 0649 |  |
|  |  | 55922 | 15:32:02 | 15:40:02 | 0650 |  |
|  |  | 56402 | 15:40:02 | 15:48:02 | 0651 |  |
|  |  | 56882 | 15:48:02 | 15:56:02 | 0652 |  |
|  |  | 57362 | 15:56:02 | 16:04:02 | 0653 |  |
|  |  | 57842 | 16:04:02 | 16:12:02 | 0654 |  |
|  |  | 58322 | 16:12:02 | 16:20:02 | 0655 |  |
| 16:28:02 | End recording of Triton occultation | 58802 | 16:20:02 | 16:28:02 | 0656 |  |
|  |  | 59282 | 17:00:00 | 17:05:00 | 0657 | Test |
|  |  | 59762 | 17:10:00 | 17:15:00 | 0658 | Test |

Table 2. Voyager 2 Triton occultation time line with open loop data tape numbers (column 6).

File and Record Formats:

A typical tape contains a 32-byte tape header followed by 8 minutes of data in 24000 data records of 4166 bytes each.  Each data record comprises 166 bytes of header data (Figure 1) and 4000 8-bit receiver samples (Figure 2).  An example of RSC-11-10A binary data accompanies this document in the PDS4 archive; also included are translations of the binary to ASCII.

Important: RSC-11-10A binary data were written in “most significant byte” (MSB) first order.  Most contemporary computers (ca. 2025) store and read numbers in “least significant byte” (LSB) order, so it is important to handle original RSC-11-10A data at the byte level to prevent corruption of values by unintentional “byte swapping”.

Figure 1a. Structure of RSC-11-10A record header (Words 1-39).
One 16-bit word (left margin) is two 8-bit bytes.

Figure 1b. Structure of RSC-11-10A record header (Words 40-77).
One 16-bit word (left margin) is two 8-bit bytes.

Figure 1c. Structure of RSC-11-10A record header (Words 78-83).
One 16-bit word (left margin) is two 8-bit bytes.

Figure 2. Structure of the data portion of RSC-11-10A records.  The record header ends at Word 83  For Voyager 2 at Neptune, receiver samples were 8-bits and N=2083.  Each 16-bit word (left margin) is two 8-bit bytes.

Example Data File:

An example of RSC-11-10A data (nc0590a.dat) generated at Canberra during the Voyager 2 encounter with Neptune accompanies this document.  The example file is a truncated version of original file NC0590, shown starting at 02:35:02 in Table 1.  Figure 3 shows the first 240 bytes in hexadecimal format from the unix-like command

od -t x1 nc0590a.dat +0. | head -15

0000000    44  53  50  52  2d  35  32  30  35  2d  4f  50  2d  44  2d  56

0000016    37  2e  31  33  00  00  00  00  00  00  00  00  00  00  00  00

0000032    d1  01  00  01  08  23  28  00  20  28  b2  ed  00  8d  ef  f0

0000048    54  45  53  54  2a  31  20  20  41  20  75  43  29  79  11  84

0000064    84  84  00  8d  ec  08  00  43  29  79  11  84  84  84  00  8d

0000080    ec  26  00  00  00  01  79  3d  bc  ce  50  00  69  3d  bc  cc

0000096    50  00  11  10  00  8d  ef  fa  00  00  00  00  00  00  00  00

0000112    00  00  ff  fe  d9  bb  33  33  33  66  30  30  6f  75  00  00

0000128    00  00  00  8d  8c  f4  01  dd  03  2a  00  02  00  05  00  00

0000144    00  00  00  00  00  00  00  8d  8c  f4  05  6b  04  48  05  87

0000160    04  7b  f1  1b  00  01  00  01  d8  2f  00  01  00  01  e9  16

0000176    00  01  00  01  db  28  00  01  00  01  00  8d  ec  08  c3  50

0000192    a5  5a  0e  fa  36  11  49  8f  72  50  a8  9c  83  67  39  6f

0000208    78  89  98  67  95  79  2c  85  86  81  a3  75  99  96  8b  9b

0000224    7b  93  84  67  bc  b9  73  67  51  78  89  75  9a  7a  80  67

Figure 3. Hexadecimal dump of the first 240 bytes from file nc0590a.dat.  The byte counter along the left margin is in decimal.  The first 32 bytes (yellow highlighting) contain the tape header, the next 166 bytes (green highlighting) contain the first record header.  The first data sample is in byte 199.

A full hexadecimal dump of the example file is available (with annotations) in  nc0590a.hex.pdf

Tape Header: The 32-byte tape header is a string of ASCII characters, which translates to

DSPR-5205-OP-D-V7.13

followed by 12 ASCII null characters (0x00).

Record Headers: The first record header may be unpacked as shown in Table 3; see [1] for interpretations.  Headers from all five data records are given in accompanying file nc0590a.hdr.  If nc0590a.hdr were read into a spreadsheet (such as Microsoft Excel), file nc0590a.txt provides field titles which can be added to the spreadsheet.

| Table 3. Unpacked Record Header from Data Record 1 (nc0590a.dat) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bit Field | Byte Location | Bit Location (from start of record) | Value (b = binary) (h = hexadecimal) | Description | Unpacked Value |
| 1 | 1 | 1 | 1b | Time tag and configuration flag | 1 |
| 2 | 1 | 2 | 1b | Start of recording flag | 1 |
| 3 | 1 | 3 | 0b | Error flag | 0 |
| 4 | 1 | 4 | 1b | Sample resolution | 0 |
| 5 | 1 | 5-8 | 1h | Compression factor | 1 |
| 6 | 2 | 9-16 | 01h | Tape number | 1 |
| 7 | 3-4 | 17-32 | 0001h | Record number | 1 |
| 8 | 5-6 | 33-48 | 0823h | Record length (words) | 2083 |
| 9 | 7 | 49-56 | 28h | Prime front end area (FEA) | 40 |
| 10 | 8 | 57-64 | 00h | Secondary FEA | 0 |
| 11 | 9 | 65-72 | 20h | Spacecraft number | 32 |
| 12 | 10 | 73-80 | 28h | Signal Processing Center (SPC) | 40 |
| 13 | 11 | 81-87 | 1011001b | Year (last two digits) | 89 |
| 14 | 11-12 | 88-96 | edh | Day of year | 237 |
| 15 | 13 | 97-101 | 00h | Unused 1 | 0 |
| 16 | 13-16 | 97-128 | 8deff0h | Time of first sample (milliseconds past 0h UTC) | 9302000 |
| 17 | 17-26 | 129-208 | 544553542a3120204120h | Predict set ID | TEST*1 A |
| 18 | 27 | 209 | 0b | POCA status (source of control) | 0 |
| 19 | 27 | 210 | 1b | POCA status (readiness) | 1 |
| 20 | 27 | 211 | 1b | POCA status (synthesizer power) | 1 |
| 21 | 27 | 212 | 1b | POCA status (synthesizer lock status) | 1 |
| 22 | 27 | 213 | 0b | POCA status (limit enable) | 0 |
| 23 | 27 | 214 | 1b | POCA status (track) | 1 |
| 24 | 27 | 215 | 0b | POCA status (acquisition) | 0 |
| 25 | 27 | 216 | 1b | POCA status (sweep) | 1 |
| 26 | 28-34 | 217-272 | 43297911848484h | POCA readback frequency (BCD, mHz) | 43297911848484 |
| 27 | 35 | 273-277 | 00h | Unused 2 | 0 |
| 28 | 35-38 | 273-304 | 8dec08h | FTS readback time (msec past 0h UTC) | 9301000 |
| 29 | 39 | 305-312 | 00h | Unused 3 | 0 |
| 30 | 40-46 | 313-368 | 43297911848484h | POCA calculated frequency (BCD, mHz) | 43297911848484 |
| 31 | 47 | 369-373 | 00h | Unused 3 | 0 |
| 32 | 47-50 | 374-400 | 8dec26h | Diagnostic POCA time tag (msec past 0h UTC) | 9301030 |
| 33 | 51 | 401-402 | 0b | Antenna configuration code (selected) | 0 |
| 34 | 51 | 403-404 | 0b | Antenna configuration code (reported) | 0 |
| 35 | 51 | 405-408 | 0h | Unused 4 | 0 |
| 36 | 52-54 | 409-428 | 00000h | POCA frequency rate (BCD, tenths of Hz/s) | 0 |
| 37 | 54 | 429-431 | 000b | Power of 10 multiplier for POCA frequency rate | 0 |
| 38 | 54 | 432 | 1b | Sign for frequency rate (0=negative, `1=positive) | 1 |
| 39 | 55-60 | 433-480 | 793dbcce5000h | One-second accumulated phase #1 (2^-20 cycles) | 133306067603456 |
| 40 | 61-66 | 481-528 | 693dbccc5000h | One-second accumulated phase #2 (2^-20 cycles) | 115713881427968 |
| 41 | 67 | 529-532 | 0001b | FMS test facility input signal selection | 1 |
| 42-45 | 67 | 533-536 | 0001b | FMS sample control register | 1 |
| 46 | 68 | 537-540 | 0001b | Frequency counter 1 mode register | 1 |
| 47 | 68 | 541-544 | 0000b | Frequency counter 2 mode register | 0 |
| 48 | 69 | 545-549 | 00h | Unused 5 | 0 |
| 49 | 69-72 | 550-576 | 8deffah | FMS counter time tag (msec) | 9302010 |
| 50 | 73-74 | 577-585 | 000000000b | Predict time offset | 0 |
| 51 | 74 | 586-590 | 00h | Unused 6 | 0 |
| 52 | 74 | 591 | 0b | Sign of predict time offset (1=negative, 0=positive) | 0 |
| 53 | 74-76 | 592-608 | 00000h | Predict time offset (sec) | 0 |
| 54 | 77-82 | 609-656 | 000000000000h | S-band predict frequency offset (2^-20 Hz) | 0 |
| 55 | 83-86 | 657-688 | fffed9bbh | S-band filter offset (Hz) | -75333 |
| 56 | 87 | 689-692 | 3h | RIC filter for Channel 1 (selected) | 3 |
| 57 | 87 | 691-696 | 3h | RIC filter for Channel 2 (selected) | 3 |
| 58 | 88 | 697-700 | 3h | RIC filter for Channel 3 (selected) | 3 |
| 59 | 88 | 701-704 | 3h | RIC filter for Channel 4 (selected) | 3 |
| 60 | 89 | 705-708 | 3h | RIC filter for Channel 1 (reported) | 3 |
| 61 | 89 | 709-712 | 3h | RIC filter for Channel 2 (reported) | 3 |
| 62 | 90 | 713-716 | 6h | RIC filter for Channel 3 (reported) | 6 |
| 63 | 90 | 717-720 | 6h | RIC filter for Channel 4 (reported) | 6 |
| 64 | 91 | 721-728 | 30h | RIV attenuator setting for Channel 1 (dB) | 48 |
| 65 | 92 | 729-736 | 30h | RIV attenuator setting for Channel 2 (dB) | 48 |
| 66 | 93 | 737-744 | 6fh | RIV attenuator setting for Channel 3 (dB) | 111 |
| 67 | 94 | 745-752 | 75 | RIV attenuator setting for Channel 4 (dB) | 117 |
| 68 | 95 | 753-760 | 00h | RIV attenuator for future use | 0 |
| 69 | 96 | 761-768 | 00h | RIV attenuator for future use | 0 |
| 70 | 97 | 769-776 | 00h | RIV attenuator for future use | 0 |
| 71 | 98 | 777-784 | 00h | RIV attenuator for future use | 0 |
| 72 | 99 | 785-789 | 00h | Unused 7 | 0 |
| 73 | 99-102 | 790-816 | 8d8cf4h | RIV attenuator time tag (msec) | 9276660 |
| 74 | 103-104 | 817-832 | 1ddh | RIC RMS voltage, Channel 1 (mv) | 477 |
| 75 | 105-106 | 833-848 | 32a h | RIC RMS voltage, Channel 2 (mv) | 810 |
| 76 | 107-108 | 849-864 | 2h | RIC RMS voltage, Channel 3 (mv) | 2 |
| 77 | 109-110 | 865-880 | 5h | RIC RMS voltage, Channel 4 (mv) | 5 |
| 78 | 111-112 | 881-896 | 0 | Future RMS voltage #1 | 0 |
| 79 | 113-114 | 897-912 | 0 | Future RMS voltage #2 | 0 |
| 80 | 115-116 | 913-928 | 0 | Future RMS voltage #3 | 0 |
| 81 | 117-118 | 929-944 | 0 | Future RMS voltage #4 | 0 |
| 82 | 119 | 945-949 | 0 | Unused 8 | 0 |
| 83 | 119-122 | 950-976 | 8d8cf4h | RIC RMS voltage time tag (msec) | 9276660 |
| 84 | 123-124 | 977-992 | 56bh | Software calculated RMS voltage, Channel 1 (mv) | 1387 |
| 85 | 125-126 | 993-1008 | 448h | Software calculated RMS voltage, Channel 2 (mv) | 1096 |
| 86 | 127-128 | 1009-1024 | 587h | Software calculated RMS voltage, Channel 3 (mv) | 1415 |
| 87 | 129-130 | 1025-1040 | 47bh | Software calculated RMS voltage, Channel 4 (mv) | 1147 |
| 88 | 131 | 1041-1048 | f1h | Maximum ADC value, Channel 1 | 241 |
| 89 | 132 | 1049-1056 | 1bb | Minimum ADC value, Channel 1 | 27 |
| 90 | 133-134 | 1057-1072 | 0001h | Number of occurrences of value in byte 131 | 1 |
| 91 | 135-136 | 1077-1088 | 0001h | Number of occurrences of value in byte 132 | 1 |
| 92 | 137 | 1089-1096 | d8h | Maximum ADC value, Channel 2 | 216 |
| 93 | 138 | 1097-1104 | 2fh | Minimum ADC value, Channel 2 | 47 |
| 94 | 139-140 | 1105-1120 | 0001h | Number of occurrences of value in byte 137 | 1 |
| 95 | 141-142 | 1121-1136 | 0001h | Number of occurrences of value in byte 138 | 1 |
| 96 | 143 | 1137-1144 | e9h | Maximum ADC value, Channel 3 | 233 |
| 97 | 144 | 1145-1152 | 16h | Minimum ADC value, Channel 3 | 22 |
| 98 | 145-146 | 1153-1168 | 0001h | Number of occurrences of value in byte 143 | 1 |
| 99 | 147-148 | 1169-1184 | 0001h | Number of occurrences of value in byte 144 | 1 |
| 100 | 149 | 1185-1192 | dbh | Maximum ADC value, Channel 4 | 219 |
| 101 | 150 | 1193-1200 | 28h | Minimum ADC value, Channel 4 | 40 |
| 102 | 151-152 | 1201-1216 | 0001h | Number of occurrences of value in byte 149 | 1 |
| 103 | 153-154 | 1217-1232 | 0001h | Number of occurrences of value in byte 150 | 1 |
| 104 | 155 | 1233-1237 | 0 | Unused 9 | 0 |
| 105 | 155-158 | 1238-1264 | 8dec08h | Time tag of max-min samples (msec) | 9301000 |
| 106 | 159-160 | 1265-1280 | c350h | Single ADC sample rate (samples per second) | 50000 |
| 107 | 161-162 | 1281-1296 | a55ah | NBOC sync code | A55A |
| 108 | 163 | 1297-1304 | 0eh | “24” counter | 14 |
| 109 | 164 | 1305-1312 | fah | “N” register | 250 |
| 110 | 165 | 1313 | 0b | NBOC overflow flag | 0 |
| 111 | 165 | 1314 | 0 | Unused 10 | 0 |
| 112 | 165 | 1315 | 1b | NBOC PLL in-lock flag | 1 |
| 113 | 165 | 1316 | 1b | Sample rate flag | 1 |
| 114 | 165 | 1317 | 0b | Operational mode flag | 0 |
| 115 | 165 | 1318 | 1b | Sample resolution flag | 1 |
| 116 | 165 | 1319-1320 | 10b | Sample mode flag | 2 |
| 117 | 166 | 1321-1322 | 00b | AD1 assignment | 0 |
| 118 | 166 | 1323-1324 | 01b | AD2 assignment | 1 |
| 119 | 166 | 1325-1326 | 00b | AD3 assignment | 0 |
| 120 | 166 | 1327-1328 | 01b | AD4 assignment | 1 |

Data Values: The first data record in the example file includes 4000 8-bit data samples.  In Figure 3, receiver samples begin with hexadecimal value 0x49 (decimal 73) in byte 199.  This sample as well as the fifth, ninth, etc. came from AD1; the third, seventh, eleventh, etc. came from AD3.  Together, they provide the sample stream that represents the output from the Channel 1 (X-RCP) receiver.  The second, fourth, sixth, etc. samples are from AD2 and AD4; they combine to represent the output from the Channel 2 (S-RCP) receiver (see accompanying files nc0590x.tab and nc0590s.tab respectively).  Figure 4 shows histograms of the sample values from the two receivers.

Figure 4. Histograms of the samples from each receiver in data records 1-5 of the example file (10000 samples in each histogram).  Receiver 1 (X-RCP, left) is represented by samples from AD1 and AD3; Receiver 2 (S-RCP, right) is represented by samples from AD2 and AD4.

G. L. Tyler, D. N. Sweetnam, J. D. Anderson, S. E. Borutzki, J. K. Campbell, V. R. Eshleman, D. L. Gresh, E. M. Gurrola, D. P. Hinson, N. Kawashima, E. R. Kursinski, G. S. Levy, G. F. Lindal, J. R. Lyons, E. A. Marouf, P. A. Rosen, R. A. Simpson, and G. E. Wood. Voyager Radio Science Observations of Neptune and Triton, Science, 246, 1466-1473, 1989.