Interpretation and Use of Binary RSC-11-9 Data

This document describes and illustrates extraction of values from binary files generated according to the NASA Deep Space Network (DSN) RSC-11-9 Software Interface Specification (SIS).  In this document, RSC-11-9 is referenced as:

[1] Document 820-013 (Rev. A), DSN System Requirements, Detailed Interface Design, 
RSC-11-9, DSN Radio Science System Interface, Original Data Record, 
effective date 1 March 1985 (PDS logical identifier urn:nasa:pds:radiosci.documentation:dsn.rsc-11-9)

RSC-11-9 is one of several modules within DSN 820-013 that governed generation of open loop radio science raw data products over about two decades starting in the late 1970s.  RSC-11-9 specifies the format and content of the most important open loop radio science files collected using NASA Deep Space Network antennas near Canberra, Australia, during the Voyager 2 encounter with Uranus.  A variation of this format, used for collection of data during the Voyager 2 encounter with Uranus at Parkes Observatory (Australia), is described separately.  Its citation differs only in the year of the effective date.

[2] Document 820-013 (Rev. A), DSN System Requirements, Detailed Interface Design, RSC-11-9, DSN Radio Science System Interface, Original Data Record, effective date 1 March 1982 (PDS logical identifier urn:nasa:pds:radiosci.documentation:dsn.rsc-11-9p)

Data Overview:

The data were originally distributed on Computer-Compatible Tapes (CCTs).  In the mid-1990s the tape data were copied to CD-ROM volumes, and in 2020 the CD-ROMs were copied to a laptop.  Subsequent handling has been carried out on the electronic (laptop) files.  Throughout, [1] has remained the descriptive document of the Canberra data.

Open loop data were the primary inputs for the study of radio occultations by the neutral atmosphere, ionosphere, and rings of Uranus.  These digital data were generated by the Spectrum Processing Assembly (SPA) of the Deep Space Communications Complex (DSCC) Spectrum Processing Subsystem (DSP), using radio science software of the DSP, designated “DSP-R”.

During the Uranus encounter, S- and X-band receiver outputs in right-circular polarization (RCP) from the 64-m antenna (DSS43) and the 34-m antenna (DSS42) at the DSCC near Canberra, Australia, were written to tape at 50000 8-bit (real) samples per second from each of the four receiver channels.  Table 1 shows the Uranus encounter timeline, significant events, and the corresponding tape numbers.  Table 2 lists recordings of test data.

Table 1. This table lists significant radio science events during the Voyager 2 encounter with Uranus (as seen in Australia) as a function of time.  Tapes which were delivered to the Voyager Radio Science Team are listed in column 8 (UC tapes).  Backup tapes, created at Stanford University, are shown in column 9.  Tapes containing test data are listed in Table 2.

Table 2. Open loop test data collected after the Voyager 2 Uranus encounter.  Tapes delivered to the Voyager Radio Science Team are listed in column 8 (UC Tape).  Backup tapes, created at Stanford University, are listed in column 9.  Notation  mmmm:n  denotes backup tape number  mmmm  and file number  n  (starting from 0).

File and Record Formats:

A typical tape contains a 32-byte tape header followed by 8 minutes of data in 24000 records of 4390 bytes each.  Each data record comprises 80 bytes of header data (Figure 1), 4000 8-bit receiver samples (Figure 2), 300 bytes of Precision Power Monitor (PPM) data (Figure 3), and 10 bytes of operator-entered offsets (Figure 4).  An example of RSC-11-9 binary data accompanies this document in the PDS4 archive; also included are translations of the binary to ASCII.

Important: RSC-11-9 binary data were written in “most significant byte” (MSB) first order.  Most contemporary computers (ca. 2024) store and read numbers in “least significant byte” (LSB) order, so it is important to handle original RSC-11-9 data at the byte level to prevent corruption of values by unintentional “byte swapping”.

Figure 1. Structure of RSC-11-9 record header.  One 16-bit word (left margin) is two 8-bit bytes.

Figure 2. Structure of the RSC-11-9 data area.  One 16-bit word (left margin) is two 8-bit bytes.  For Voyager 2 at Uranus, the offset of the data area was  N=2040  words and the analog-to-digital converter assignments were AD1=DSS-43 S-RCP, AD2=DSS-42 S-RCP, AD3=DSS-43 X-RCP, and AD4=DSS-42 X-RCP.

Figure 3. Structure of the RSC-11-9 PPM area.  One 16-bit word (left margin) is two 8-bit bytes. 
For Voyager 2 at Uranus, the offset was  N=2040 words.

Figure 4. Structure of the RSC-11-9 operator-entered offset (OEO) area.  One 16-bit word (left margin) is two 8-bit bytes.  For Voyager 2 at Uranus, the offset was  N=2040 words.

Example Data File:

An example of RSC-11-9 data (uc0452a.dat) generated at Canberra during the Voyager 2 encounter with Uranus accompanies this document in the PDS4 archive.  Data were collected at the DSN ground station near Canberra, Australia, on 1986-024.  The example file is a truncated version of original file UC0452, shown at 21:15:01 in Table 1.  Figure 5 shows the first 240 bytes in hexadecimal format from the unix command

od -t x1 uc0452a.dat +0. | head -15

0000000    44  4d  4f  2d  35  32  30  35  2d  4f  50  2d  42  20  56  20

0000016    34  2e  33  20  00  00  00  00  00  00  00  00  00  00  00  00

0000032    c1  01  00  01  08  93  2b  00  20  28  ac  18  04  8f  50  08

0000048    5a  52  20  20  20  20  20  20  41  20  75  43  35  03  04  10

0000064    46  54  04  8f  50  08  00  43  35  03  04  10  61  02  84  8f

0000080    50  12  00  43  93  81  00  00  33  1f  1f  e5  00  00  33  1f

0000096    1f  e2  1d  10  04  8f  50  08  c3  50  a5  5a  12  f9  14  1b

0000112    8b  64  85  8a  b4  94  61  89  78  8c  70  86  7c  a5  95  81

0000128    aa  a6  90  8a  6b  7a  82  86  6c  71  90  88  91  77  8e  8a

0000144    92  8e  9f  89  89  85  a1  8a  8e  85  93  8a  80  aa  6c  8c

0000160    b2  93  7a  87  9f  8f  bb  81  77  8d  98  8c  7d  82  8f  94

0000176    7e  71  7f  8b  9c  b2  77  8a  8f  cf  9d  87  95  74  a7  88

0000192    8c  72  88  89  64  58  4f  8d  8b  82  80  86  71  a7  79  88

0000208    8f  6b  8b  84  b4  98  92  8d  7e  82  89  91  6e  7d  a2  88

0000224    98  76  8f  87  a2  99  96  8b  8f  93  95  84  84  81  7f  8e

0000240                               ...

Figure 5. Hexadecimal dump of the first 240 bytes from file uc0452a.dat.  The byte counter along the left margin is given in decimal.  The first 32 bytes (yellow highlighting) contain the tape header, the next 80 bytes (green highlighting) contain the first record header.  The first data sample is in byte 113.

A full hexadecimal dump of the example file is available (with annotations) in  uc0452a.hex.pdf

Tape Header: The 32-byte tape header is a string of ASCII characters, which translates to

DMO-5205-OP-B V 4.3

followed by one ASCII space character (0x20) and 12 ASCII null characters (0x00).

Record Headers: The first record header may be unpacked as shown in Table 3.  Asterisks (*) in the right column denote flag values; see [1] for interpretations.  Headers from all ten records are given in accompanying file uc0452a.hdr.  If uc0452a.hdr were read into a spreadsheet (such as Microsoft Excel), file uc0452a.txt provides field headers which can be added to the spreadsheet.

| Table 3. Unpacked Record Header from Data Record 1 (uc0452a.dat) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bytes | Bit Numbers | Value (b = binary) (h = hexadecimal) | Description | Unpacked Value |
| 1 | 1 | 1b | Time tag and configuration flag | 1* |
| 1 | 2 | 1b | Start of recording flag | 1* |
| 1 | 3 | 0b | Error flag | 0* |
| 1 | 4 | 0b | Sample resolution | 0* |
| 1 | 5-8 | 1h | Compression factor | 1* |
| 2 | 9-16 | 01h | Tape number | 1 |
| 3-4 | 17-32 | 0001h | Record number | 1 |
| 5-6 | 33-48 | 0893h | Record length (words) | 2195 |
| 7 | 49-56 | 2bh | Prime front end area (FEA) | 43 |
| 8 | 57-64 | 00h | Secondary FEA | 0 |
| 9 | 65-72 | 20h | Spacecraft number | 32 |
| 10 | 73-80 | 28h | Signal Processing Center (SPC) | 40 |
| 11 | 81-87 | 56h | Year (last two digits) | 86 |
| 11-12 | 88-96 | 18h | Day of year | 24 |
| 13-16 | 97-128 | 048f5008h | Time of first sample (milliseconds past 0h UTC) | 76501000 |
| 17-26 | 129-208 | 5a522020202020204120h | Predict set ID | ZR A |
| 27 | 209 | 0b | POCA status (source of control) | 0* |
| 27 | 210 | 1b | POCA status (readiness) | 1* |
| 27 | 211 | 1b | POCA status (synthesizer power) | 1* |
| 27 | 212 | 1b | POCA status (synthesizer lock status) | 1* |
| 27 | 213 | 0b | POCA status (limit enable) | 0* |
| 27 | 214 | 1b | POCA status (track) | 1* |
| 27 | 215 | 0b | POCA status (acquisition) | 0* |
| 27 | 216 | 1b | POCA status (sweep) | 1* |
| 28-34 | 217-272 | 43350304104654h | POCA readback frequency (BCD, mHz) | 43350304104654 |
| 35-38 | 273-304 | 048f5008h | FTS readback time (milliseconds past 0h UTC) | 76501000 |
| 39-46 | 305-368 | 0043350304106102h | POCA calculated frequency (BCD, mHz) | 43350304106102 |
| 47 | 369-372 | 8h | Receiver filter | 8 |
| 47-50 | 373-400 | 48f5012h | Diagnostic POCA time tag (milliseconds past 0h UTC) | 76501010 |
| 52-54 | 409-428 | 43938h | POCA rate (BCD, Hz per second) | 43938 |
| 54 | 429-431 | 000b | POCA rate power of ten | 0 |
| 54 | 432 | 1b | POCA rate sign | 1* |
| 55-60 | 433-480 | 0000331f1fe5h | 1-s S phase, counter 1 (scaled to 2-8 cycles) | 857677797 |
| 61-66 | 481-528 | 0000331f1fe2h | 1-s S phase, counter 2 (scaled to 2-8 cycles) | 857677794 |
| 67 | 529-532 | 1h | Test signal selection | 1 |
| 67 | 533-536 | dh | Sample control register | 13 |
| 68 | 537-540 | 1h | Frequency counter 1 mode register | 1 |
| 68 | 541-544 | 0h | Frequency counter 2 mode register | 0 |
| 69-72 | 545-576 | 048f5008h | FMS time tag (milliseconds past 0h UTC) | 76501000 |
| 73-74 | 577-592 | c350h | Single ADC sample rate | 50000 |
| 75-76 | 593-608 | a55ah | NBOC sync code | a55ah |
| 79 | 625 | 0b | NBOC overflow flag | 0* |
| 79 | 628 | 1b | Sample rate flag | 1* |
| 79 | 629 | 0b | Test mode flag | 0* |
| 79 | 630 | 1b | 8-bit sample flag | 1* |
| 79 | 631-632 | 00b | ADC assignment flag | 00* |
| 80 | 633-634 | 00b | AD1 assignment | 00b |
| 80 | 635-636 | 01b | AD2 assignment | 01b |
| 80 | 637-638 | 10b | AD3 assignment | 10b |
| 80 | 639-640 | 11b | AD4 assignment | 11b |

Data Values: The first record in the example file includes 4000 8-bit data samples.  In Figure 5, samples begin with hexadecimal value 0x8b (decimal 139) in byte 113.  This sample as well as the fifth, ninth, etc. came from AD1.  The second, sixth, tenth, etc. samples came from AD2; the third, seventh, etc. came from AD3; and the fourth, eighth, etc. came from AD4 (Figure 2).  If the data records are parsed in this way, the original sample stream can be recovered (see accompanying files uc0452a1.tab, uc0452a2.tab, uc0452a3.tab, and uc0452a4.tab, respectively).  Figure 7 shows histograms of the sample values from each of the A-D converters in records 1-10 of the example file.  Figure 8 shows power in AD outputs.  Note that the histograms have similar shapes and the means are within 1 unit; but the distribution from AD4 is about 5 times narrower (and the histogram peak is about 5 time larger) than for the other three converters.

Figure 7. Histograms of the samples from each analog-to-digital converter in records 1-10 of the example file.  The histograms are very similar except that the peak value for AD4 is about 5 times larger and its standard deviation is about 5 times smaller than for the other histograms.

Figure 8. Power versus time in the samples from each analog-to-digital converter (ADC) versus time.  Each sample value was reduced by the mean for its channel, the result was squared, and 101 such values were averaged to produce one plotted point.  Sample values were extracted from the first 10 data records of UC0452; so there were 10000 original samples and 100 plotted points for each ADC.

Tapes UC0463, UC0482, UC0486, and UC0490 contain 24050 records (one extra second).  Tapes UC0451 and UC0493 are short, containing 7800 and 23750 records, respectively.  Tapes with test data contain variable numbers of records.

The expected value in this field is hexadecimal  bh  or decimal  11 [1].